Equipment Pool No.: 001Operational daily rates in DMRate in DMless 11 days11-45 daysover 45 daysTransitMob/Demob (Lump Sum)1500,00*1000,00*800,00*500,00*4125,00** This offer is based on our prevailing general contract conditions

System for geological or biological exploration of the deep ocean floor, applicable also in rough floor morphology. Photos and real time TV pictures are usually taken in

3-5 m distance from the seafloor. In addition, continuous measurements of scientific parameters can be taken and transmitted to the shipboard units.

Operation of OFOS

heaving and lowering speed, approx. 0.8 m/s (which corresponds to

20-25 minutes per 1000 m water depth)

recommended operation time at seafloor: up to 4-6 hours (accumulator capacity is typically sufficient for this amount of time, depending upon power consumption of flood lights)

recharging of an accumulator set requires approx. 12 hours, therefore at least

2 sets of accumulators are recommended

The system is presently equipped with:

Low light level TV camera

Flood lights

Photo camera, capacity 800 pictures

Deep-sea flash lights

Ground distance sonar

CTD probe (measurement of conductivity, temperature and depth )

2 deep-sea batteries (12 V, 230 Ah each)

Technical Data:

Operation depth6000 m

Size1.6 m (L) x 1.2 m (W) x 1.4 m (H)

Weight                  520 kg, in air

350 kg, in water