Equipment Pool No.: 027Operational rates in DMRates in DMPrice/deploymentTransit/dayMob/Demob (Lump Sum)1.000,-/each

230,-/each

180,-/each105,-/each  1.200,-/each*2.500,-/5 pieces

980,-/5 pieces

780,-/5 pieces105,-/5 pieces3.000,-/5 pieces*4.500,-/10 pieces

4500,-/10pieces

1430,-/10pieces105,-/10 pieces5.000,-/10 pieces *Consumables not included* This offer is based on our prevailing general contract conditions

The OBH/OBS system is

designed for seismic

exploration of the sea floor subsurface. For this purpose traveltimes of active sound sources (eg. airguns) or

passive sources (earth-quakes) are recorded at the sea floor in the range of 3 Hz up to 50 Hz. Modelling of travel-times for detected events reveals a 2-D or 3-D picture of the observed subsurface velocity field.

Major features of the system are:

Design and outfit according to the specific requirements of the experiment,

uniform construction, modular structure using tested components, easy maintenance, large data capacity using digital recording, low power consumption, window programmable, quick access to data after recovery of the system, easy handling, quick assembly, minimal personal requirement, easy deployment from any ship. Up to 24 instruments can be accomodated and transported in a 20' standard

container.

The system consists of the following components:

Underwater-unit:

Central frame with flotation, flag, flash and radio beacon, receiving hydrophone, digital recording unit (DAT tape or flash memory), acoustic release transponder with additional time release, anchor and 3 component seismometer (optional).

In the lab:

PC for programming, GPS clock for synchronizing and reference, deckunit for acoustic release system, direction finder.

Onboard processing system (optional, needs training)

For active sound source experiments airguns are recommended. 32 l and 0.8 - 2.0 l airguns are available. Passive sound source experiments (earthquake monitoring) have been successfully accomplished as well. Typically up to 24 instruments are used within one field experiment.

The operation is characterized by up to eight major steps:

1Depth test of the release transponders attached to a deep-sea cable (once per cruise)

2Programming of the backup time release

3Programming and synchronizing of the recorders

4Deployment

5Active sound source operation/wait time for passive sources

(up to 30 days)

6Acoustic release and system recovery

7Check time reference of recording unit

8Playback if processing system is available

Technical Data:

Max. possible operating depth6.000 m

Residual buoyance5 kg

Return velocity1.0 m/s

Temperatur Range0°C bis + 40 °C

Recording capacityup to 1.08 Gigabyte

Channels (independent)1 - 4

Duration of the continuous recording2 days lead accumulator or

at max. current (4 channels, 5 ms)14 days alkali cells

References:	Scientific Technical Expedition Services

1996	FS Sonne, San Franciso - Astoria

5 Weeks	SO-108: Mob/Demob, Logistic, Equipment on board: OBS-System, Airguns

Customer: BMFT /GEOMAR

1996	FS Sonne, Balboa - San Francisco

5 Weeks	SO-107: Mob/Demob, Logistic, Equipment on board: OBS-System, Airguns

Customer: BMFT /GEOMAR

1995	FS Sonne, Santiago-Lima

6 Weeks	SO-105: Mob/Demob, Logistic, Equipment on board: OBH-System, Airguns

Customer: Uni Hamburg

1994	FS Sonne, Valparaiso-Valparaiso

4 Weeks	SO-104: Mob/Demob, Logistic, Equipment on board: OBS-System, Airguns

Customer: BMFT / GEOMAR

1994	FS Sonne, Kodiak - Kodiak

5 Weeks	SO-96: Mob/Demob, Logistic, Equipment on board: OBS/OBH

Customer:  BMFT / GEOMAR

1993	FS Meteor, Piräus - malta

5 Weeks	M24-4: Mob/Demob, Logistic, Equipment on board: OBS/OBH

Customer:  BMFT / GEOMAR

1993	FS Meteor, Las palmas - Malaga

4 Weeks	M24 Ozeanvulkan: Mob/Demob, Logistic, Equiment on board: Bottom Sampler,

OBS/OBH

Customer:  DFG / GEOMAR

1992	FS Sonne, Caldera - Caldera

2 Weeks	SO-81: Mob/Demob, Logistic, Equipment on board: OBS/OBH

Customer:  BMFT / GEOMAR

1991-92	FS Sonne, Balboa - Balboa

6 Weeks 	SO-76: Mob/Demob, Logistic, Equipment on board: OBS/OBH

Customer:  BMFT / GEOMAR